TECHNICAL SPECIFICATION

Construction of Cast-in-Situ RCC Pile Foundations for 220 kV Double Circuit Transmission Line Towers

1.0 Scope of Work

The scope of work shall comprise the complete design (where applicable), construction, testing, quality control, and successful completion of cast-in-situ reinforced cement concrete (RCC) bored pile foundations for transmission line towers in accordance with the approved drawings, technical specifications, relevant IS Codes, and directions of the Engineer-in-Charge.

The work shall include, but not be limited to, the following:

- Mobilization of all manpower, construction equipment, machinery, tools and plants.

- Site clearance, jungle cutting, preparation of working platform, temporary access roads, temporary island/platform (where required), earth filling, dewatering and all temporary works necessary for execution.

- Survey, setting out and marking of pile locations.

- Boring of cast-in-situ piles through all types of strata including clay, silt, sand, gravel, boulders, weathered rock, hard rock and mixed strata by suitable rotary/hydraulic piling rigs.

- Installation and withdrawal of temporary casing wherever required for maintaining bore stability.

- Supply, fabrication and installation of MS liner of minimum 6 mm thickness for the specified length wherever shown in the approved drawings.

- Use of bentonite/polymer slurry or any other approved drilling fluid for stabilization of boreholes wherever required.

- Cleaning of boreholes before concreting.

- Conducting Standard Penetration Test (SPT), bore logging and other geotechnical observations wherever specified.

- Supply, fabrication and placement of reinforcement cages using Fe-500D or higher grade TMT bars.

- Concreting of piles using M25 Grade Design Mix Concrete through tremie method wherever necessary to ensure uninterrupted and defect-free concreting.

- Chipping of pile heads to remove laitance concrete up to sound concrete level.

- Construction of RCC pile caps/pile heads including excavation, shuttering, reinforcement, concreting, curing and backfilling.

- Disposal of excavated earth and construction debris to approved locations.

- Dewatering during excavation and concreting operations.

- Protection and curing of completed concrete.

- Testing of completed piles including:

- Pile Integrity Test (PIT)

- Low Strain Integrity Test

- Initial Load Test (where specified)

- Routine Load Test (where specified)

- Submission of test reports, quality records and as-built drawings.

- Restoration of the site after completion of works.

2.0 Technical Requirements

2.1 Pile Type

- Bored Cast-in-Situ Reinforced Cement Concrete Piles.

- Nominal Diameter: 1200 mm

- Number of piles: As per approved foundation drawings.

- Safe Working Load: As specified in the approved design.

2.2 Boring

The Contractor shall carry out boring using suitable piling rigs capable of penetrating all anticipated soil and rock strata.

The boring operation shall include:

- Temporary guide casing.

- Drilling through all types of soil and rock.

- Bore stabilization.

- Removal of muck.

- Bore cleaning before reinforcement placement.

- Maintenance of verticality.

The permissible deviation in verticality shall not exceed 1 in 150.

2.3 Temporary Casing / MS Liner

Where specified, the Contractor shall provide and install:

- Mild Steel Liner

- Thickness: 6 mm minimum

- Length: As indicated in drawings.

- Proper welding and alignment.

2.4 Reinforcement

Reinforcement shall conform to:

- IS 1786

- Grade Fe-500D or higher.

The reinforcement cage shall be fabricated accurately to maintain:

- Diameter

- Cover

- Verticality

- Rigidity during lowering.

Adequate spacers shall be provided.

2.5 Concrete

Concrete shall conform to IS 456.

Minimum Grade:

M25 Design Mix

Concrete shall be supplied from approved batching plants or produced using weigh batching.

Concrete shall have:

- Adequate workability

- Approved slump

- Proper vibration (where applicable)

- Continuous placement without interruption.

Where tremie concreting is adopted, the tremie pipe shall remain embedded inside fresh concrete throughout pouring.

2.6 Pile Head

After completion of concreting:

- Remove laitance.

- Chip the pile head up to sound concrete.

- Ensure proper bond with pile cap.

No payment shall be made for laitance concrete.

2.7 Pile Cap

Construction shall include:

- Excavation

- Dewatering

- PCC (where required)

- Reinforcement

- Shuttering

- M25 RCC

- Vibration

- Curing

- Backfilling

3.0 Quality Assurance

The Contractor shall establish a Quality Assurance Plan covering:

- Material approval

- Cement testing

- Aggregate testing

- Reinforcement testing

- Concrete mix approval

- Slump test

- Cube strength test

- Bore log records

- Reinforcement inspection

- Concreting records

- Test reports

No work shall proceed without approval of inspection stages.

4.0 Testing

The following tests shall be carried out wherever specified:

- Standard Penetration Test (SPT)

- Pile Integrity Test (PIT)

- Low Strain Integrity Test

- Initial Load Test

- Routine Load Test

- Concrete Cube Test

- Reinforcement Testing

- Cement Testing

All testing shall comply with relevant IS Codes.

5.0 Applicable Standards

The latest editions of the following standards shall apply:

- IS 2911 (Part 1 & Part 4) – Design and Construction of Pile Foundations

- IS 456 – Plain and Reinforced Concrete

- IS 1786 – High Strength Deformed Steel Bars

- IS 383 – Coarse and Fine Aggregates

- IS 10262 – Concrete Mix Design

- IS 516 – Methods of Tests for Concrete

- IS 1892 – Subsurface Investigation

- IS 6403 – Bearing Capacity of Foundations

- IS 875 – Design Loads

- IS 2062 – Structural Steel

- Relevant CEA Regulations

- AEGCL Construction Standards

6.0 Measurement

Measurement shall be made as follows:

- Bored piles: Per running metre of completed pile measured from the bottom of the pile cap to the approved founding level.

- Reinforcement: Per kilogram of reinforcement actually incorporated.

- MS Liner: Per kilogram.

- RCC in pile head/pile cap: Per cubic metre.

- Mobilization: Per Job.

7.0 Rates

The quoted rates shall be deemed to include:

- Mobilization and demobilization

- Temporary works

- Site preparation

- Labour

- Machinery

- Fuel

- Water

- Electricity

- Dewatering

- Boring

- Bentonite/polymer slurry

- Temporary casing

- Formwork

- Reinforcement fabrication and fixing

- Concrete production and placement

- Curing

- Quality control

- Testing (including SPT and pile integrity tests unless separately specified)

- Safety measures

- Environmental protection

- Disposal of excavated material

- All duties, insurance and incidental expenses required for complete execution of the work.